# LAB 4 : Introduction to Packet Tracer: Basic Router Configuration

Objective(s)

• To understand and practice basic router and switch configuration commands in Cisco Packet Tracer.
• To secure router access using passwords and encryption.
• To configure interfaces, VLAN, and verify connectivity between devices.

Background

Packet Tracer is a powerful network simulator that can be utilized in training for network

certification like and learning by allowing students to create networks with an almost

unlimited number of devices and to experience troubleshooting without having to buy

real Cisco routers or switches. The tool is created by Cisco Systems. The purpose of Packet

Tracer is to offer students a tool to learn the principles of networking.

| Prompt | Mode Description |
| --- | --- |
| Router> | User mode |
| Router# | Privileged mode |
| Router(config)# | Global configuration mode |
| Router(config-if)# | Interface mode |
| Router(config-subif)# | Subinterface mode |
| Router(config-line)# | Line mode |
| Router(config-router)# | Router configuration mode |

Router Configuration

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#hostname Nishan

Nishan (config)#enable password cisco

Nishan(config)#enable secret class

Nishan(config)#line console 0

Nishan(config-line)#password cisco

Nishan(config-line)#login

Nishan(config-line)#exit

Nishan(config)#line vty 0 4

Nishan(config-line)#password cisco

Nishan(config-line)#login

Nishan(config-line)#exit

Nishan(config)#service password-encryption

Nishan(config)#exit

Nishan#

Nishan#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Nishan(config)#service password-encryption

Nishan(config)#exit

Nishan#

%SYS-5-CONFIG_I: Configured from console by console

Nishan#clock ?

set Set the time and date

Nishan#clock set?

set

Nishan#clock set ?

hh:mm:ss Current Time

Nishan#clock set 11:11:11 ?

<1-31> Day of the month

MONTH Month of the year

Nishan#clock set 11:11:11 oct

% Incomplete command.

Nishan#clock set 11:11:11 oct ?

<1-31> Day of the month

Nishan#clock set 11:11:11 oct 30 ?

<1993-2035> Year

Nishan#clock set 11:11:11 oct 30 2004

Nishan#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Nishan(config)#banner motd $ UNATHORISED ACCESS RESTRICTED $

Nishan(config)#interface GigabitEthernet0/0/0

Nishan(config-if)#ip address 192.168.1.1 255.255.255.0

Nishan(config-if)#ip address 192.168.1.1 255.255.255.0

Nishan(config-if)#no shutdown

Nishan(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0, changed state to up

Nishan(config-if)#exit

Nishan(config)#exit

Nishan#

%SYS-5-CONFIG_I: Configured from console by console

Nishan#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

SWITCH CONFIGURATION

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#interface vlan 1

Switch(config-if)#ip address 192.168.1.4 255.255.255.0

Switch(config-if)#no shutdown

# LAB 5: Static Routing Implementation

### Objective

- Understand how Static Routing works

- Learn its advantages and disadvantages

- Configure static routes on routers

## Background: Static Routing

Static routing is manually configured routing. It is ideal for small networks where the number of routes is limited.

### Advantages

- Simple to configure

- Very secure (no routing updates shared)

- No CPU or memory overhead

### Disadvantages

- Not scalable for large networks

- Cannot auto-reroute traffic when a link fails

# Basic Configuration of Routers

Router: NISHAN1

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#hostname NISHAN1

NISHAN1(config)#

NISHAN1(config)#enable password cisco

NISHAN1(config)#enable secret class

NISHAN1(config)#line console 0

NISHAN1(config-line)#password cisco

NISHAN1(config-line)#login

NISHAN1(config-line)#line vty 0 15

NISHAN1(config-line)#password cisco

NISHAN1(config-line)#login

NISHAN1(config-line)#line aux 0

NISHAN1(config-line)#password cisco

NISHAN1(config-line)#login

NISHAN1(config-line)#exit

NISHAN1(config)#service password-encryption

Router Nishan2:

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#hostname NISHAN2

NISHAN2(config)#

NISHAN2(config)#ENABLE password cisco

NISHAN2(config)#enable secret class

NISHAN2(config)#

NISHAN2(config)#line console 0

NISHAN2(config-line)#password cisco

NISHAN2(config-line)#login

NISHAN2(config-line)#

NISHAN2(config-line)#line vty 0 15

NISHAN2(config-line)#password cisco

NISHAN2(config-line)#login

NISHAN2(config-line)#

NISHAN2(config-line)#line aux 0

NISHAN2(config-line)#password cisco

NISHAN2(config-line)#login

NISHAN2(config-line)#exit

NISHAN2(config)#service password-encryption

NISHAN2(config)#

NISHAN2#

%SYS-5-CONFIG_I: Configured from console by console

2. Router Interface Configuration

Router 1(NISHAN1)

Serial Link

NISHAN1(config)#interface Serial0/1/0

NISHAN1(config-if)#description

NISHAN1(config-if)#description Link to NISHAN2

NISHAN1(config-if)#ip address 192.168.1.1 255.255.255.0

NISHAN1(config-if)#clock rate 64000

NISHAN1(config-if)#no shutdown

Fast Ethernet

NISHAN1(config)#interface FastEthernet0/0

NISHAN1(config-if)#ip address 192.168.2.1 255.255.255.0

NISHAN1(config-if)#no shutdown

Router 2(NISHAN2)

Serial Link

NISHAN1(config)#interface Serial0/1/0

NISHAN1(config-if)#description

NISHAN1(config-if)#des link from LAN to internet

NISHAN1(config-if)#ip address 192.168.1.2 255.255.255.0

NISHAN1(config-if)#clock rate 64000

NISHAN1(config-if)#no shutdown

Fast Ethernet

NISHAN2(config)#interface FastEthernet0/0

NISHAN2(config-if)#ip address 192.168.3.1 255.255.255.0

NISHAN2(config-if)#no shutdown

3. Routes Configuration

Router 1(NISHAN1)

NISHAN1(config)#ip route 192.168.3.0 255.255.255.0 192.168.1.2

Router 2(NISHAN2)

NISHAN2(config)#ip route 192.168.2.0 255.255.255.0 192.168.1.1

4.Result

# LAB 6: VLAN and VLAN Trunking.

Objective(s):

- Understand LAN segmentation using VLANs

- Create VLANs on multiple switches

- Configure Access Ports and Trunk Ports

- Assign IPs to hosts in different VLANs

- Verify inter-switch VLAN connectivity

Background

What is a VLAN?

A Virtual Local Area Network (VLAN) divides a physical network into logical sub-networks.
Devices in the same VLAN behave as if they are on the same LAN—even if connected to different switches.

Access Ports

- Connect end devices (PCs, printers)

- Carry traffic for only one VLAN

- Frames are not tagged

Trunk Ports

- Connect switches to switches

- Carry traffic for all VLANs

- Frames are tagged to specify VLAN

- Required for VLAN communication across switches

Native VLAN

- Default VLAN on a trunk (usually VLAN 1)

- Frames on native VLAN are not tagged

1.Configuration VLAN on Both Switches

Switch 1

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname SW1B

SW1B(config)#

SW1B(config)#vlan 10

SW1B(config-vlan)#name BLACK

SW1B(config-vlan)#exit

SW1B(config)#vlan 20

SW1B(config-vlan)#name GRAY

SW1B(config-vlan)#exit

SW1B(config)#vlan 30

SW1B(config-vlan)#name CYAN

SW1B(config-vlan)#exit

SW1B(config)#exit

SW1B#show vlan brief

Switch 2

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname SW2B

SW2B(config)#

SW2B(config)#vlan 10

SW2B(config-vlan)#name BLACK

SW2B(config-vlan)#EXIT

SW2B(config)#vlan 20

SW2B(config-vlan)#name GRAY

SW2B(config-vlan)#exit

SW2B(config)#vlan 30

SW2B(config-vlan)#name CYAN

SW2B(config-vlan)#EXIT

SW2B(config)#EXIT

SW2B#show vlan brief

2. Configure Access Mode on both the Switches

Switch 1

SW1B(config)#interface FastEthernet0/2

SW1B(config-if)#switchport mode access

SW1B(config-if)#switchport access vlan 10

SW1B(config-if)#interface fastethernet 0/3

SW1B(config-if)#switchport mode access

SW1B(config-if)#switchport access vlan 20

SW1B(config-if)#interface fastethernet 0/4

SW1B(config-if)#switchport mode access

SW1B(config-if)#switchport access vlan 30

SW1B(config-if)#exit

SW1B(config)#exit

SW1B#show vlan brief

Switch2

SW2B(config)#interface FastEthernet0/2

SW2B(config-if)#switchport mode access

SW2B(config-if)#switchport access vlan 10

SW2B(config-if)#interface fastethernet 0/3

SW2B(config-if)#switchport mode access

SW2B(config-if)#switchport access vlan 20

SW2B(config-if)#interface fastethernet 0/4

SW2B(config-if)#switchport mode access

SW2B(config-if)#switchport access vlan 30

SW2B(config-if)#exit

SW2B(config)#exit

SW2B#

%SYS-5-CONFIG_I: Configured from console by console

SW2B#show vlan brief

3. Configure the Trunk Mode – Configure the mode trunk to all interface

of the switches that connects to another switches

switch1

SW1B(config)#interface FastEthernet0/1

SW1B(config-if)#switchport mode trunk

SW1B(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

SW1B(config-if)#exit

SW1B(config)#exit

Switch 2

SW2B(config)#interface FastEthernet0/1

SW2B(config-if)#switchport mode trunk

SW2B(config-if)#switchport nonegotiate

SW2B(config-if)#exit

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

SW2B(config)#exit

4. Configure Trunk on Native VLAN 1 on both switch

Switch1

SW1B(config)#interface FastEthernet0/24

SW1B(config-if)#switchport mode trunk

SW1B(config-if)#switchport trunk native vlan 1

SW1B(config-if)#exit

Switch2

SW2B(config)#interface FastEthernet0/24

SW2B(config-if)#switch mode trunk

SW2B(config-if)#switchport trunk native vlan 1

SW2B(config-if)#exit

5. Configure the IP address and subnet mask on the PCs as follows. There is no layer three device on the network so the default gateway will not be configured.

VLAN 10: 192.168.10.0/24

VLAN 20: 192.168.20.0/24

VLAN 30: 192.168.30.0/24

PC0: 192.168.10.110

PC2:192.168.20.220

PC1: 192.168.30.30

PC3: 192.168.10.111

PC4: 192.168.20.221

PC5: 192.168.30.31

Result:

# LAB 7: Implementation of RIP, EIGRP, OSPF and BGP

Objective

To implement and understand the operation of the following routing protocols:

- RIP (Routing Information Protocol)

- EIGRP (Enhanced Interior Gateway Routing Protocol)

- OSPF (Open Shortest Path First)

- BGP (Border Gateway Protocol)

Background:

Routing Protocols are the set of defined rules used by routers to communicate between source and destination networks. They do not directly move the information from the source to the destination but help update and maintain the routing table, which contains all necessary path information. Routing protocols determine the best possible route for packet forwarding based on metrics such as hop count, cost, bandwidth, delay, and other parameters.

Routing protocols are broadly classified into IGP (Interior Gateway Protocols) and EGP (Exterior Gateway Protocols).

# Router Configuration

# Serial Link Configuration

# Router 1

# Router(config)#hostname  Nishan

# Nishan(config)#interface s0/0/0

# Nishan (config-if)#ip address 192.168.5.1 255.255.255.0

# Nishan (config-if)#clock rate 64000

# Nishan (config-if)#bandwidth 1024

# Nishan (config-if)#no shutdown

# Router 2

# Router(config)#hostname  NishanR2

# NishanR2(config)# interface serial 0/0/0

# NishanR2(config-if)#ip address 192.168.5.2 255.255.255.0

# NishanR2(config-if)#no shutdown

# NishanR2(config-if)#exit

# Other Interfaces

# Router 1

# Nishan (config)#interface fastethernet 0/0

# Nishan (config-if)#ip address 192.168.1.1 255.255.255.0

# Nishan (config-if)#no shutdown

# Nishan (config-if)#interface fastethernet 0/1

# Nishan (config-if)#ip address 192.168.2.1 255.255.255.0

# Nishan (config-if)#no shutdown

# Nishan (config-if)#exit

# Nishan (config)#exit

# Router 2

# NishanR2(config)#interface fastethernet 0/0

# NishanR2(config-if)#ip address 192.168.3.1 255.255.255.0

# NishanR2(config-if)#no shutdown

# NishanR2(config-if)#interface fastethernet 0/1

# NishanR2(config-if)#ip address 192.168.4.1 255.255.255.0

# NishanR2(config-if)#no shutdown

# NishanR2(config-if)#exit

# NishanR2(config)#exit

RIP Implementation

Router 1

Nishan(config)#router

Nishan(config-router)#network 192.168.1.0

Nishan(config-router)#network 192.168.2.0

Nishan(config-router)#network 192.168.3.0

Nishan(config-router)#network 192.168.4.0

Nishan(config-router)#network 192.168.5.0

Nishan(config-router)#exit

Nishan(config)#exit

Nishan#show run

Router 2

NishanR2(config)#router rip

NishanR2(config-router)#network 192.168.1.0

NishanR2(config-router)#network 192.168.2.0

NishanR2(config-router)#network 192.168.3.0

NishanR2(config-router)#network 192.168.4.0

NishanR2(config-router)#network 192.168.5.0

NishanR2(config-router)#exit

NishanR2(config)#exit

NishanR2#show run

Verify connections

EIGRP (Enhanced Interior Gateway Routing Protocol)

Router 1

Nishan(config)#no router rip

Nishan(config)#router eigrp 343

Nishan(config-router)#network 192.168.1.0

Nishan(config-router)#network 192.168.2.0

Nishan(config-router)#network 192.168.3.0

Nishan(config-router)#network 192.168.4.0

Nishan(config-router)#network 192.168.5.0

Nishan(config-router)#exit

Router 2

NishanR2(config)#no router rip

NishanR2(config)#router eigrp 343

NishanR2(config-router)#network 192.168.1.0

NishanR2(config-router)#network 192.168.2.0

NishanR2(config-router)#network 192.168.3.0

NishanR2(config-router)#network 192.168.4.0

NishanR2(config-router)#network 192.168.5.0

NishanR2(config-router)#exit

OSPF Implementation

Router 1

Nishan(config)#no router eigrp 343

Nishan(config)#router ospf 343

Nishan (config-router)#network 192.168.1.0 0.0.0.255 area 0

Nishan (config-router)#network 192.168.2.0 0.0.0.255 area 0

Nishan (config-router)#network 192.168.3.0 0.0.0.255 area 0

Nishan (config-router)#network 192.168.4.0 0.0.0.255 area 0

Nishan (config-router)#network 192.168.5.0 0.0.0.255 area 0

Nishan (config-router)#exit

Router 2

NishanR2(config)#no router eigrp 343

NishanR2(config)#router ospf 343

NishanR2(config-router)#network 192.168.1.0 0.0.0.255 area 0

NishanR2(config-router)#network 192.168.2.0 0.0.0.255 area 0

NishanR2(config-router)#network 192.168.3.0 0.0.0.255 area 0

NishanR2(config-router)#network 192.168.4.0 0.0.0.255 area 0

NishanR2(config-router)#network 192.168.5.0 0.0.0.255 area 0

NishanR2(config-router)#exit

LAB 8: Firewall Implementation, Router Access Control List (ACL)

Objective(s):

- To understand the concept of router firewalls using Access Control Lists (ACLs).

- To implement packet filtering using Standard and Extended ACLs on Cisco routers.

Background:

At the network layer, packet filtering is commonly achieved using Access Control Lists (ACLs) configured on routers. When ACLs are applied, the router functions like a basic firewall by selectively permitting or denying packets based on defined rules.

ACLs allow network administrators to:

- Control unwanted network traffic.

- Restrict access to specific hosts or networks.

- Enhance security by blocking unauthorized communication.

ACLs inspect packets based on:

- Source IP/network

- Destination IP/network

- Protocols (IP, TCP, UDP)

- Ports or services (for extended ACLs)

Without ACLs, routers permit all traffic by default. Filtering occurs only when ACL rules are applied.

Types of ACLs

1. Standard ACL

Standard ACLs filter traffic based only on the source IP address.

- Valid ACL number range: 1–99

- Should be applied closest to the destination

- Can:

  - Permit or deny source IP addresses only

Standard ACLs are best when simple source-based filtering is needed.

2. Extended ACL

Extended ACLs provide more granularity. They can filter packets based on:

- Source IP Address

- Destination IP Address

- Protocol (IP/TCP/UDP/ICMP)

- Port numbers or services (HTTP, FTP, Telnet, etc.)

- Valid ACL number range: 100–199

- Should be applied closest to the source

Extended ACLs are used for fine-grained traffic control, such as blocking specific services or protocols.

Important Notes About ACLs

- ACLs may be identified using either numbers or names, depending on the protocol.

- The identifying number must fall within the valid range for that ACL type.

- ACLs consist of a series of permit or deny statements evaluated top-to-bottom.

- There is an implicit “deny any” rule at the end of every ACL, even if it is not visible.

- ACLs can filter traffic:

  - Inbound (when packets enter the interface)

  - Outbound (when packets exit the interface)

When a packet arrives at the router, the ACL determines whether the packet will be forwarded or blocked based on whether it matches the configured rules.

Configurations

LAB 09: DHCP, WEB,DNS, EMAIL, FTP Server Configuration

Router Interface Configuration

• Go to router and assign static ip addresses

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#hostname NishanR

NishanR(config)#

NishanR(config)#int g0/0

NishanR(config-if)#ip address 192.168.1.1 255.255.255.0

NishanR(config-if)#no shutdown

NishanR(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

NishanR(config-if)#int g0/1

NishanR(config-if)#ip address 192.168.2.1 255.255.255.0

NishanR(config-if)#no shutdown

NishanR(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to up

DHCP helper configuration in the Router

• Enable ip helper-address in the router for DHCP

NishanR(config)#interface GigabitEthernet0/0

NishanR(config-if)#ip helper-address 192.168.2.2

NishanR(config-if)#

DHCP Server Configuration

• Assign the static ip address to the server

• Turn ON the DHCP service and create the server pool as follows

Assign Dynamic IP Configuration to PC0 and PC1

• Go to PC0(Nishan)>Desktop>IP Configuration then select DHCP

• Go to PC1(Nishan)>Desktop>IP Configuration then select DHCP

HTTP Server

• Assign static ip address to the HTTP server

• Make sure to turn ON the HTTP Service

DNS Server Configuration

• Assign the static ip address to the DNS server

• Add DNS Records of the domain name (eg. www.testserver.com)

• Go to PC0>Desktop>Web browser and open the url

EMAIL Server Configuration

• Assign the static ip address to the EMAIL server

• Turn ON the SMTP and POP3 services and create two users

• Go to PC0>Email>Configure user as follows

• Go to PC1>Email>Configure user as follows

• Go to PC0>Desktop>Email and compose the email as follows

• Go to PC1>Desktop>Email and check (receive) email as follows

FTP Server Configuration

• Assign static ip address to the FTP server as follows

• Enable the FTP service and create two users: admin (with write ,read, delete, rename, list options) and user (with read and list only)

• Go to PC0>Desktop>Text Editor, create a file testb1.txt and save it on the desktop

• Go to Desktop>Command Prompt and verify the created file using dir command then login into the ftp server as follows using admin user and password

• Use PUT command to upload test.txt into the FTP Server as follows:

Verify the uploaded file using the command dir

Go to PC1>Desktop>Command Prompt, connect to the FTP server using user account and password and use GET command to download file test.txt into the desktop. After the transfer type dir command to verify it.